PACIFIC MARINE & INDUSTRIAL®

P.O. Box 70520 ◊ Richmond, California, United States ◊ 94807-0520

Phone: 510-233-2310 ◊ Fax: 510-233-2322 ◊ Web: www.PacificMarine.net

THROUGHPUT CAPACITIES    		1/h

Rated Capacity                              	                2,900

Recommended max. throughput*

Fuel oils — diesel engine

Distillate

- Viscosity 1.5 - 5.5 cSt/4000      		                2,300

Marine diesel oil

- Viscosity 13 cSt/400C                	     2,000

Lubricating oils,

By-pass treatment; optimum

Detergent

-	crosshead diesel		                                 1,100

- trunk piston diesel 				800

______________________________

*	Maximum density of oil 991 kg/m3 at 15◦C. Density preferably measured at 50◦C and according to ASTM method D 1298-80, corrected to 15◦C accoriding to ASTM tables D 1250-80.

SLUDGE AND WATER SPACE		.32 litres

BUILT-IN OIL OUTLET PARING DISC PUMP

ELECTRIC MOTOR                                            Size 3.0 kW

POWER CONSUMPTION

- idling			.9 kW

- MDO                                          2.0kW

SPEED

The prescribed speed of the bowl spindle, which must not be exceeded, is stamped on the name plate of the machine.

Table of rpm at 	50 Hz	60 Hz

- Drive motor              3000         3600
	 -  Bowl spindle             max, 9510

STARTING—UP/STOPPING TIME

Starting time:     approx. 2 mins.

Stopping time:   15-20 mins

MATERIALS

Frame, lower and upper parts

- cast iron *

Frame hood

cast iron (grey finish)*

Bowl body and hood, disc stack, gravity discs, sliding bowlbottom;

Disc stack, gravity disc, heavy phase cover, wing insert
	 - stainless steel

Other Bowl Parts

brass

Oil Paring disc

brass

Other inlet and outlet parts

- brass, stainless steel, cast iron

_______________________

*	An epoxy enamel

OPERATING WATER

Total Water Cons. Per discharge:  3 Litres

Total hardness: max. 180 ppm CaCO3 (l00dH)

pH -value: > 6

Salinity:  	max. cloride content of 60 ppm Cl 				(Equivalent to 100 ppm NaC1)

LUBE OIL IN GEAR		.5 litres

SHIPPING DATA

Separator, complete

Weights: net 230 kg, gross 280 kg

Volume: 1.06 m3